ESP8266 CODE:

/*

============================================================

REMOTE PATIENT MONITORING — ESP8266

Sensors: MAX30102 (HR/SpO2), MPU6050 (Accel)

Fix: MAX30102 "not found" no longer halts — system continues

============================================================

*/

#include <Wire.h>

#include "MAX30105.h"

#include "spo2_algorithm.h"

#include <MPU6050.h>

#include <ESP8266WiFi.h>

#include <ESP8266HTTPClient.h>

#include <LiquidCrystal_I2C.h>

#include <time.h>

/* ================= LCD ================= */

LiquidCrystal_I2C lcd(0x27, 16, 2);

/* ================= SENSORS ================= */

MAX30105 sensor;

MPU6050  mpu;

// ── FIX: track whether each sensor was found ──────────────────────

bool sensorMAX_ok = false;   // true only if MAX30102 initialised OK

bool sensorMPU_ok = false;   // true only if MPU6050 found

// ──────────────────────────────────────────────────────────────────

#define BUFFER_SIZE 50

uint32_t irBuffer[BUFFER_SIZE];

uint32_t redBuffer[BUFFER_SIZE];

int32_t  spo2          = 0;

int8_t   validSpO2     = 0;

int32_t  heartRate     = 0;

int8_t   validHeartRate = 0;

int16_t ax = 0, ay = 0, az = 0;

float   activityLevel  = 0;

/* ================= PATIENT DATA ================= */

String patientName      = "";

String patientAge       = "";

String patientCondition = "";

String patientEmail     = "";

bool   patientEntered   = false;

/* ================= WIFI ================= */

const char* ssid     = "Redmi note 12 pro+ 5G";

const char* password = "suprathik";

/* ================= SERVER ================= */

const char* SERVER_IP   = "10.249.152.137";

const int   SERVER_PORT = 3000;

String serverURL = String("http://") + SERVER_IP + ":" + SERVER_PORT + "/sensor";

/* ================= SMOOTHING BUFFERS ================= */

#define SMOOTH_SIZE 5

int hrBuf[SMOOTH_SIZE]   = {0};

int spo2Buf[SMOOTH_SIZE] = {0};

int bufIndex = 0;

bool bufFull = false;

/* ================= TIMEOUT ================= */

const unsigned long WIFI_TIMEOUT = 20000;

const unsigned long HTTP_TIMEOUT = 8000;

/* ============================================================

MOVING AVERAGE

============================================================ */

int smoothedHR() {

int sum = 0, count = 0;

int n = bufFull ? SMOOTH_SIZE : bufIndex;

for (int i = 0; i < n; i++) { if (hrBuf[i] > 0) { sum += hrBuf[i]; count++; } }

return count > 0 ? sum / count : 0;

}

int smoothedSpO2() {

int sum = 0, count = 0;

int n = bufFull ? SMOOTH_SIZE : bufIndex;

for (int i = 0; i < n; i++) { if (spo2Buf[i] > 0) { sum += spo2Buf[i]; count++; } }

return count > 0 ? sum / count : 0;

}

/* ============================================================

TIME

============================================================ */

String getIndianTime() {

time_t now = time(nullptr);

struct tm* t = localtime(&now);

char buf[30];

sprintf(buf, "%02d-%02d-%04d %02d:%02d:%02d",

t->tm_mday, t->tm_mon + 1, t->tm_year + 1900,

t->tm_hour, t->tm_min,  t->tm_sec);

return String(buf);

}

/* ============================================================

SERIAL PATIENT REGISTRATION

============================================================ */

void getPatientFromSerial() {

Serial.println("\n========================================");

Serial.println("   PATIENT REGISTRATION");

Serial.println("========================================");

Serial.println("Enter Patient Name:");

while (!Serial.available()) delay(10);

patientName = Serial.readStringUntil('\n');

patientName.trim();

Serial.println("Enter Age:");

while (!Serial.available()) delay(10);

patientAge = Serial.readStringUntil('\n');

patientAge.trim();

Serial.println("Enter Medical Condition:");

while (!Serial.available()) delay(10);

patientCondition = Serial.readStringUntil('\n');

patientCondition.trim();

Serial.println("Enter Guardian Email:");

while (!Serial.available()) delay(10);

patientEmail = Serial.readStringUntil('\n');

patientEmail.trim();

Serial.println("\n Patient Registered:");

Serial.println("  Name : " + patientName);

Serial.println("  Age  : " + patientAge);

Serial.println("  Cond : " + patientCondition);

Serial.println("  Email: " + patientEmail);

Serial.println("========================================\n");

patientEntered = true;

showPatientLCD();

delay(3000);

}

/* ============================================================

LCD HELPERS

============================================================ */

void showPatientLCD() {

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Pt: " + patientName.substring(0, 12));

lcd.setCursor(0, 1); lcd.print("Age: " + patientAge);

delay(2000);

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Condition:");

lcd.setCursor(0, 1); lcd.print(patientCondition.substring(0, 16));

delay(2000);

// Show sensor status on LCD

lcd.clear();

lcd.setCursor(0, 0);

lcd.print(sensorMAX_ok ? "MAX: OK" : "MAX: NOT FOUND");

lcd.setCursor(0, 1);

lcd.print(sensorMPU_ok ? "MPU: OK" : "MPU: MISSING");

delay(2500);

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Ready! Place");

lcd.setCursor(0, 1); lcd.print("Finger on Sensor");

}

void showVitalsLCD(int hr, int sp, float act) {

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("HR:");

lcd.print(hr);

lcd.print(" SpO2:");

lcd.print(sp);

lcd.print("%");

lcd.setCursor(0, 1);

lcd.print((hr > 100 || hr < 60) ? "ALERT" : "OK");

}

/* ============================================================

SEND DATA TO SERVER

============================================================ */

void sendData(int hr, int sp, float activity) {

if (WiFi.status() != WL_CONNECTED) {

Serial.println("WiFi lost — reconnecting...");

unsigned long t = millis();

while (WiFi.status() != WL_CONNECTED && millis() - t < WIFI_TIMEOUT) {

delay(300); Serial.print(".");

}

if (WiFi.status() != WL_CONNECTED) {

Serial.println("\nWiFi reconnect failed");

lcd.clear(); lcd.print("WiFi Failed!");

return;

}

Serial.println("\nWiFi reconnected");

}

WiFiClient client;

HTTPClient http;

client.setTimeout(HTTP_TIMEOUT / 1000);

http.begin(client, serverURL);

http.addHeader("Content-Type", "application/json");

http.setTimeout(HTTP_TIMEOUT);

String json = "{";

json += "\"name\":\""      + patientName        + "\",";

json += "\"age\":\""       + patientAge         + "\",";

json += "\"condition\":\"" + patientCondition   + "\",";

json += "\"heartRate\":"   + String(hr)         + ",";

json += "\"spo2\":"        + String(sp)         + ",";

json += "\"activity\":"    + String(activity, 2)+ ",";

json += "\"ax\":"          + String(ax / 16384.0, 4) + ",";

json += "\"ay\":"          + String(ay / 16384.0, 4) + ",";

json += "\"az\":"          + String(az / 16384.0, 4) + ",";

json += "\"email\":\""     + patientEmail       + "\"";

json += "}";

Serial.println("\n Sending -> " + serverURL);

Serial.println(" Payload : " + json);

int code = http.POST(json);

Serial.print(" Response: "); Serial.println(code);

if (code == 200 || code == 201) {

Serial.println(" Server ACK: " + http.getString());

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Sent OK!");

lcd.setCursor(0, 1); lcd.print("HR:" + String(hr) + " SpO2:" + String(sp));

} else if (code < 0) {

Serial.println(" Connection failed: " + http.errorToString(code));

lcd.clear(); lcd.print("No Connection");

lcd.setCursor(0, 1); lcd.print("Check Server IP");

} else {

Serial.println(" HTTP Error: " + String(code));

lcd.clear(); lcd.print("HTTP Err:" + String(code));

}

http.end();

delay(500);

}

/* ============================================================

SETUP

============================================================ */

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\n=====================================");

Serial.println(" REMOTE PATIENT MONITORING SYSTEM");

Serial.println("=====================================");

Wire.begin(D2, D1);  // SDA=D2, SCL=D1

lcd.init();

lcd.backlight();

lcd.clear();

lcd.print("Starting...");

// ── WiFi ────────────────────────────────────────────────────────

Serial.print("Connecting to " + String(ssid) + " ");

WiFi.begin(ssid, password);

unsigned long t = millis();

while (WiFi.status() != WL_CONNECTED && millis() - t < WIFI_TIMEOUT) {

delay(400); Serial.print(".");

}

if (WiFi.status() == WL_CONNECTED) {

Serial.println("\n WiFi OK — IP: " + WiFi.localIP().toString());

lcd.clear(); lcd.print("WiFi: ");

lcd.print(WiFi.localIP().toString().substring(0, 10));

delay(1500);

} else {

Serial.println("\n WiFi FAILED");

lcd.clear(); lcd.print("WiFi Error!");

delay(3000);

}

configTime(19800, 0, "pool.ntp.org");

delay(1000);

// Patient registration via Serial

getPatientFromSerial();

// ── MAX30102 init ────────────────────────────────────────────────

Serial.println("Initializing MAX30102...");

if (sensor.begin(Wire, I2C_SPEED_FAST)) {

sensorMAX_ok = true;

sensor.setup(0x3F, 4, 2, 400, 411, 4096);

sensor.setPulseAmplitudeRed(0x3F);

sensor.setPulseAmplitudeIR(0x3F);

Serial.println(" MAX30102 OK");

} else {

// ── FIX: do NOT halt — continue without the sensor ──────────

sensorMAX_ok = false;

Serial.println(" MAX30102 NOT FOUND — check wiring:");

Serial.println("   SDA → D2 (GPIO4)");

Serial.println("   SCL → D1 (GPIO5)");

Serial.println("   VCC → 3.3V  (NOT 5V!)");

Serial.println("   GND → GND");

Serial.println("   Add 4.7kΩ pull-ups on SDA & SCL if missing");

Serial.println(" Continuing WITHOUT pulse oximeter...");

lcd.clear();

lcd.setCursor(0, 0); lcd.print("MAX30102 Missing");

lcd.setCursor(0, 1); lcd.print("Check Wiring!");

delay(3000);

// ────────────────────────────────────────────────────────────

}

// ── MPU6050 init ─────────────────────────────────────────────────

if (mpu.testConnection()) {

mpu.initialize();

sensorMPU_ok = true;

Serial.println(" MPU6050 OK");

} else {

sensorMPU_ok = false;

Serial.println(" MPU6050 not found — accelerometer disabled");

}

Serial.println("\n Server URL: " + serverURL);

Serial.println(" Sensor status — MAX30102:" + String(sensorMAX_ok ? "OK" : "MISSING")

+ "  MPU6050:" + String(sensorMPU_ok ? "OK" : "MISSING"));

lcd.clear();

if (sensorMAX_ok) {

lcd.print("Place Finger...");

lcd.setCursor(0, 1); lcd.print("Waiting...");

} else {

lcd.print("No Pulse Sensor");

lcd.setCursor(0, 1); lcd.print("Sending data...");

}

}

/* ============================================================

MAIN LOOP

FIX: Runs even when MAX30102 is missing — sends accel data

============================================================ */

void loop() {

if (!patientEntered) { delay(500); return; }

// Read accelerometer if available

if (sensorMPU_ok) {

mpu.getAcceleration(&ax, &ay, &az);

} else {

ax = ay = az = 0;

}

// ── Case 1: MAX30102 present — normal sensing loop ──────────────

if (sensorMAX_ok) {

long irVal = sensor.getIR();

if (irVal < 7000) {

Serial.println("No finger detected (IR=" + String(irVal) + ") — waiting...");

lcd.clear(); lcd.setCursor(0, 0); lcd.print("Place Finger");

lcd.setCursor(0, 1); lcd.print("IR: " + String(irVal));

delay(2000);

return;

}

Serial.println("\nFinger detected (IR=" + String(irVal) + ") — reading " + String(BUFFER_SIZE) + " samples...");

lcd.clear(); lcd.print("Reading...");

for (byte i = 0; i < BUFFER_SIZE; i++) {

while (!sensor.available()) sensor.check();

redBuffer[i] = sensor.getRed();

irBuffer[i]  = sensor.getIR();

sensor.nextSample();

if (i % 10 == 0) {

lcd.setCursor(0, 1);

lcd.print("Samples: " + String(i) + "/" + String(BUFFER_SIZE) + "  ");

}

}

maxim_heart_rate_and_oxygen_saturation(

irBuffer, BUFFER_SIZE, redBuffer,

&spo2, &validSpO2, &heartRate, &validHeartRate

);

Serial.println("Raw — HR:" + String(heartRate) + " valid:" + String(validHeartRate)

+ "  SpO2:" + String(spo2) + " valid:" + String(validSpO2));

if (validHeartRate && heartRate > 30 && heartRate < 220 &&

validSpO2      && spo2 > 70    && spo2 < 101) {

hrBuf[bufIndex]   = heartRate;

spo2Buf[bufIndex] = spo2;

bufIndex = (bufIndex + 1) % SMOOTH_SIZE;

if (bufIndex == 0) bufFull = true;

int sHR = smoothedHR();

int sSP = smoothedSpO2();

float mag = sqrt((float)ax*ax + (float)ay*ay + (float)az*az);

activityLevel = constrain((mag / 16384.0) * 50.0, 0, 100);

Serial.println("Smoothed — HR:" + String(sHR) + " SpO2:" + String(sSP)

+ " Act:" + String(activityLevel, 1)

+ " Time:" + getIndianTime());

showVitalsLCD(sHR, sSP, activityLevel);

sendData(sHR, sSP, activityLevel);

} else {

Serial.println("Invalid reading — retrying");

lcd.clear(); lcd.print("Invalid Read");

lcd.setCursor(0, 1); lcd.print("Keep finger still");

delay(2000);

}

// ── Case 2: MAX30102 missing — send accel only ─────────────────

} else {

float mag = sqrt((float)ax*ax + (float)ay*ay + (float)az*az);

activityLevel = constrain((mag / 16384.0) * 50.0, 0, 100);

// Use 0 for HR and SpO2 — server/frontend will show "–"

int sHR = 0;

int sSP = 0;

Serial.println("No pulse sensor — sending accel only");

Serial.println(" Act:" + String(activityLevel, 1)

+ " Time:" + getIndianTime());

lcd.clear();

lcd.setCursor(0, 0); lcd.print("No Pulse Sensor");

lcd.setCursor(0, 1); lcd.print("Sending data...");

sendData(sHR, sSP, activityLevel);

}

delay(3000);

}